#### Microsoft HIDSPI Specification Support on THC

The following specification describes the THC registers which are used to support MSFT HIDSPI specification (rev0.96 draft released at Nov. 2019).

## Interrupt cause register fields programmability

In HIDSPI specification, the interrupt cause register is called “Input Report Header”. The following registers are defined to provide programmability of the bit location and length for the fields that THC HW RXDMA needs to use.

### THC Interrupt Cause Register Fields Configuration Register 1 (THC_M_PRT_DEVINT_CFG_1)

Note: THC_M_PRT_DEVINT_CFG_x can only be programmed when RXDMAs, TXDMA and PIO are stopped. All the locked bits in this register can be locked when the THC_DRV_LOCK_EN bit is 1.

MEM Offset ()

Security_PolicyGroup	THC_PORT

IntelRsvd	True

Size	32 bits

Default

| Bits | Access Type | Default | Description | PowerWell |
| --- | --- | --- | --- | --- |
| 4:0 | RW/L | 5'h00 | Bit offset of Interrupt Type (THC_M_PRT_INTTYP_OFFSET) Touch device’s “interrupt Type” field’s starting bit position in the “interrupt cause register”. Default to 0 for the THC mode. Set to 0x4 for the HIDSPI protocol where the “Input Report Type” starts from bit 4. | PGD |
| 9:5 | RW/L | 5'h04 | Length of Interrupt Type (THC_M_PRT_INTTYP_LEN) Touch device’s “interrupt Type” field’s length in the “interrupt cause register”. Default to 4 for the THC mode and HIDSPI mode. | PGD |
| 31:16 | RW/L | 16'h0002 | Value of Interrupt Type Touch Data (THC_M_PRT_INTTYP_DATA_VAL) This is a bit mask where SW can set one or multiple bits in this field for THC HW to decode the “interrupt type” field as the “read_data_available” interrupt, which will be handled by the HW DMA instead of directly interrupting SW as NDDI. Default to 0x0002 for the THC mode (e.g. value 1= touch data ready, thus bit 1 is set to 1). For HIDSPI mode, driver may need to set multiple bits in this field to support multiple interrupt type with RXDMA. | PGD |
| 14:10 | RW/L | 5'h1E | Bit offset of End of Frame (THC_M_PRT_EOF_OFFSET) Touch device’s “End of Frame” field’s starting bit position in the “interrupt cause register”. Default to 0x1E(30) for the THC mode. Set to 0x8 for the HIDSPI protocol where the “Last Segment” starts from bit 8. | PGD |

### THC Interrupt Cause Register Fields Configuration Register 2 (THC_M_PRT_DEVINT_CFG_2)

Note: THC_M_PRT_DEVINT_CFG_x can only be programmed when both RXDMAs are stopped. All the locked bits in this register can be locked when the THC_DRV_LOCK_EN bit is 1.

MEM Offset ()

Security_PolicyGroup	THC_PORT

IntelRsvd	True

Size	32 bits

Default

| Bits | Access Type | Default | Description | PowerWell |
| --- | --- | --- | --- | --- |
| 4:0 | RW/L | 5'h04 | Bit offset of Microframe Size (THC_M_PRT_UFSIZE_OFFSET) Touch device’s “microframe size” field’s starting bit position in the “interrupt cause register”. Default to 4 for the THC mode. Set to 0x9 for the HIDSPI protocol where the “Input report Length” starts from bit 9. | PGD |
| 9:5 | RW/L | 5'h14 | Length of Microframe size field (THC_M_PRT_UFSIZE_LEN) Touch device’s “microframe size” field’s length in the “interrupt cause register”. Default to 0x14(20) for the THC mode and 0xF(15) in HIDSPI mode. | PGD |
| 15:12 | RW/L | 4'h0 | Unit of Microframe size (THC_M_PRT_UFSIZE_UNIT) Touch device’s “microframe size” field’s unit in power of 2 bytes. Default to 0x0 (2^0=1byte) for the THC mode. It should be set to 0x6 (2^6=64byte) in HIDSPI mode. | PGD |
| 16 | RW/L | 1’b0 | Ignore Frame Type field (THC_M_PRT_FTYP_IGNORE) If set to 1, THC ignores Touch device’s “Frame type” read from the “interrupt cause register”. THC always use the fixed type defined in THC_M_PRT_FTYP_VAL Default to 0 for the THC mode. Set to 1 for the HIDSPI protocol. |  |
| 17 | RW/L | 1’b1 | Fixed Frame Type value (THC_M_PRT_FTYP_VAL) When THC_M_PRT_FTYP_IGNORE =1, THC always use this THC_M_PRT_FTYP_VAL field as the “Frame Type” when receiving a device interrupt. 0: Proprietary Data 1: HID Report (Including Heat Map) Default for HIDSPI support. |  |
| 24 | RW/L | 1’b0 | Disable address increment on RXDMA consecutive reads for one interrupt (THC_M_PRT_RXDMA_ADDRINC_DIS) 0: increment bus address for every read packet in one device interrupt 1: disable incrementing bus address for every read packet in one device interrupt |  |
| 25 | RW/L | 1’b0 | Disable address increment on DMA consecutive writes for one TXDMA (THC_M_PRT_TXDMA_ADDRINC_DIS) 0: increment bus address for every write packet in a TXDMA frame 1: disable incrementing bus address for every write packet in a TXDMA frame |  |
| 26 | RW/L | 1’b0 | Enable streaming mode on RXDMA consecutive reads for one interrupt (THC_M_PRT_RXDMA_PKT_STRM_EN) 0: Stream mode is disabled. THC read data packet in packetized mode as in THC 1.0. 1: During RXDMA data read phase (i.e. after reading interrupt cause register), THC reads the whole microframe of data in stream mode without CS toggling. (For implementation simplification, THC HW can just skip the opcode/address/dummy phases after the first data read “packet” and mask the CS# toggling between data packets.) | Move this to post TGPR |
| 27 | RW/L | 1’b0 | Disable address increment on DMA consecutive writes for one TXDMA (THC_M_PRT_TXDMA_PKT_STRM_EN) 0: Stream mode is disabled. THC sends TXDMA data packet in packetized mode as in THC 1.0. 1: During TXDMA, THC sends a TXDMA data frame in stream mode without CS toggling. (For implementation simplification, THC HW can just skip the opcode/address phases after the first data write “packet” and mask the CS# toggling between data packets.) |  |
| 28 | RW/L | 1’b0 | THC Interrupt Polarity (THC_M_PRT_DEVINT_POL) 0: Active low interrupt as in THC 1.0. 1: enables active high device interrupt. |  |

### THC SPI Bus Write Opcode Register  (THC_M_PRT_SPI_WR_OPCODE)

Note: The THC_M_PRT_SPI_WR_OPCODE register can't be written/updated when TX/RX DMAs is running or PIO is running cycles on the bus.

MEM Offset ()

Security_PolicyGroup	THC_PORT

IntelRsvd	True

Size	32 bits

Default

| Bits | Access Type | Default | Description | PowerWell |
| --- | --- | --- | --- | --- |
| 31:24 | RW | 8'h02 | Single IO Write SPI Opcode (SPI_SIO_WR_OPCODE) Single IO Write (1-1-1) Opcode | PGD |
| 23:16 | RW | 8'hB2 | Dual IO Write SPI Opcode (SPI_DIO_WR_OPCODE) Dual IO Write (1-2-2) Opcode | PGD |
| 15:8 | RW | 8'hE2 | Quad IO Write SPI Opcode (SPI_QIO_WR_OPCODE) Quad IO Write (1-4-4) Opcode | PGD |
| 7:0 | RW | 8'hF2 | QPI Write SPI Opcode (SPI_QPI_WR_OPCODE) QPI Write (4-4-4) Opcode | PGD |

### THC SPI Bus DMA ICRRead Opcode Register  (THC_M_PRT_SPI_ICRRD_OPCODE)

Note: The THC_M_PRT_SPI_ICRRD_OPCODE register can't be written/updated when TX/RX DMAs is running or PIO is running cycles on the bus.

MEM Offset ()

Security_PolicyGroup	THC_PORT

IntelRsvd	True

Size	32 bits

Default

| Bits | Access Type | Default | Description | PowerWell |
| --- | --- | --- | --- | --- |
| 31:24 | RW | 8'h0B | Single IO Interrupt Cause Register Read SPI Opcode (SPI_SIO_ICRRD_OPCODE) Single IO Read (1-1-1) Opcode for the interrupt cause register or PIO read | PGD |
| 23:16 | RW | 8'hBB | Dual IO Interrupt Cause Register Read SPI Opcode (SPI_DIO_ICRRD_OPCODE) Dual IO Read (1-2-2) Opcode for the interrupt cause register or PIO read | PGD |
| 15:8 | RW | 8'hEB | Quad IO Interrupt Cause Register Read SPI Opcode (SPI_QIO_ICRRD_OPCODE) Quad IO Read (1-4-4) Opcode for the interrupt cause register or PIO read | PGD |
| 7:0 | RW | 8'hFB | QPI Interrupt Cause Register Read SPI Opcode (SPI_QPI_ICRRD_OPCODE) QPI Read (4-4-4) Opcode for the interrupt cause register or PIO read | PGD |

### THC SPI Bus RXDMA Data Read Opcode Register  (THC_M_PRT_SPI_DMARD_OPCODE)

Note: The THC_M_PRT_SPI_DMARD_OPCODE register can't be written/updated when TX/RX DMAs is running or PIO is running cycles on the bus.

MEM Offset ()

Security_PolicyGroup	THC_PORT

IntelRsvd	True

Size	32 bits

Default

| Bits | Access Type | Default | Description | PowerWell |
| --- | --- | --- | --- | --- |
| 31:24 | RW | 8'h0B | Single IO DMA Read SPI Opcode (SPI_SIO_DMARD_OPCODE) Single IO Read (1-1-1) Opcode for the RXDMA touch data read | PGD |
| 23:16 | RW | 8'hBB | Dual IO DMA Read SPI Opcode (SPI_DIO_DMARD_OPCODE) Dual IO Read (1-2-2) Opcode for the RXDMA touch data read | PGD |
| 15:8 | RW | 8'hEB | Quad IO DMA Read SPI Opcode (SPI_QIO_DMARD_OPCODE) Quad IO Read (1-4-4) Opcode for the RXDMA touch data read | PGD |
| 7:0 | RW | 8'hFB | QPI DMA Read SPI Opcode (SPI_QPI_DMARD_OPCODE) QPI Read (4-4-4) Opcode for the RXDMA touch data read | PGD |

Minor change on the # dummy clocks to support HIDSPI spec’s dummy size in different IO modes.

| 1:0 | RW | 2'b11 | SPI Touch IO Read Dummy Clocks (SPI_TRDC) Firmware programs this mode after discovering capabilities of Touch device. 00: 0 dummy clocks(not supported in LKF/TGP) 01: 2 dummy clocks (not supported in LKF/TGP) (changed from 1 dummy clock in ADP) 10: 4 dummy clocks(not supported in LKF/TGP) 11: 8 dummy clocks | PGD |
| --- | --- | --- | --- | --- |

Group 2: (secondary priority after TGPR-LP)

- Changing the PIO opcode to be Softsequencing, instead of HW sequencing today.

| 15:8 | RW | 8'h0 | THC SW Sequencing Bus Command (THC_SS_CMD) In SPI port, the register read and touch data read commands are mapped to SPI read cycles. The register write and bulk write commands are mapped to SPI write cycles. The other commands are not used in SPI mode. Refer to HAS section 3.3.5.1 SPI Touch Instruction Mapping for the command values |  |
| --- | --- | --- | --- | --- |

- Support sync pattern and next read opcode.