Intel QuickSPI

(HIDSPI over Touch Host Controller)

Driver Release Notes

WW12’2022

Driver Version: 4.0.0.30

- Mohammed Abdul, Haseeb (Abdul)

Contents

1.	Introduction	5

1.1	Document Scope	5

2.	Intel QuickSPI Driver Package	6

3.	Supported Configuration	7

4.	Feature implementation/Fixes	8

5.	Known Limitations	9

6.	HW Setup with HIDSPI PCB	10

7.	BIOS Menu Configuration	12

Revision history

| Driver Revision Number | Description | Revision Work Week |
| --- | --- | --- |
| 4.0.0.29 | Initial release for QuickSPI driver based on MSFT HidSpiCx | WW51, 2021 |
| 4.0.0.30 | - Q1’22 Major quarterly release of QuickSPI driver | WW12, 2022 |

Acronyms

| Term | Description |
| --- | --- |
| QuickSPI | Intel developed Windows driver for HidSpi over Touch Host Controller |
| THC | Touch Host Controller within the chipset that supports SPI based host controller interface to touch devices |
| SPI | Serial Peripheral Interface. Synchronous serial communication bus used to attach peripherals to motherboard. In the ADP-P and MTL chipset, there are two instances of THC-SPI host controllers. |
| ACPI | Advanced Configuration and Power Interface – for this specification we are assuming ACPI 5.0 |
| HidSpi | Microsoft defined specification that defines the protocol, procedure, and features for input devices to communicate using HID protocol over SPI interface |
| HidSCx | Microsoft defined Serial Class Extension framework for HID over Serial Buses. The class extension is defined for clients which implement v1.0 of hidspi protocol |
| SoC | System on Chip |
| WDK | Windows Driver Kit |
| RPL | Raptor Lake Platform |
| ADL | Alder Lake Platform |
| MTL | Meteor Lake Platform |
| WoT | Wake on Touch |

# Introduction

This document specifies the driver release notes for QuickSPI driver, which is based on Intel’s Touch Host Controller (THC) subsystem.

## Document Scope

This document was written to guide on QuickSPI Driver releases for Windows Cobalt OS on CCG platforms starting from RPL, supporting other platforms like ADL-N, MTL. This document will focus on the Driver package content, HW setup (used for validation of driver), BIOS menu config, driver changes (feature implementations/fixes) and known limitations.

# Intel QuickSPI Driver Package

Driver Version: 4.0.0.30

Content Overview

# Supported Configuration

Target Platforms:

- RPL-P/Px/Hx

- ADL-N/P

- MTL-M/P/H

Windows OS:

- Cobalt OS

EWDK:

- 22000.Co.1052

HIDPSI Panel used for validation:

- Wacom (BOM6)

SPI Interface cable:

- 10-16 pin SPI (1.8V)

# Feature implementation/Fixes

- Unified driver for ADL-P (with A1 Si)/RPL/MTL

- Driver changes to align THC register mapping for MTL platform

- WoT related fixes.

- Error handling scenarios

- THC frequency should be limited to the lowest and closest validated SPI frequency. 
7.8125MHz and 31.25MHz are the min and max validated SPI frequencies.

- Implemented the code change to avoid second PioRead, while reading the Device Descriptor.

22014560486  [Surface][Intel QuickSPI]: Driver does a second PioRead when reading Device Descriptor.

- Details of all the above fixes on top of 4.0.0.29 are available @ https://github.com/intel-innersource/drivers.platform.ipts.hspi-driver/pull/21

# Known Limitations

- WoT doesn’t work with single tap, need to use double tap or delayed drag for swipe.

- Touch is not functional in Safemode and WinPE mode.

- Single and Dual IO Mode functionalities on HIDSPI BOM6 panel are verified, but Quad mode couldn’t be verified due to panel limitation.

- Dual IO Mode functionality can be verified till max 25MHz connection speed, due to panel limitation.

HSD’s:

| id | Title |
| --- | --- |
| 16015197357 | [ADL-P/RPL-P/Px] [QuickSpi][Cobalt]::Unexpected Reset Notification from HIDSPICx during Wake On Touch (PUID 575905) |
| 16015290403 | [ADL-P][QuickSPI]YB observed on QuickSPI driver in safemode |
| 15010866040 | [ADL-P][QuickSPI][Cobalt] Touch is not functional in WinPE environment |

# HW Setup with HIDSPI PCB

- Current driver package is validated with below HW setup.

SHARP LQ0DASE170

110700425W00

3342-20Y06-1-CW70782

WACOM PWB-D893(X)

Touch Cable 10-16 pin SPI (1.8V)

| Name | Platform | Display | Touch Vendor | Touch Protocol (Firmware) | Touch Cable | eDP Cable |
| --- | --- | --- | --- | --- | --- | --- |
| BOM6 | ADL | Sharp 12.5" 1W | Wacom G14T Fusion PCBA | HID-SPI over THC | 10-16 pin SPI (1.8V) | 60-pin |

# BIOS Menu Configuration

Below values can be configured by the user from BIOS menu, which are specific to Wacom (BoM6) panel.

BIOS settings for Single IO mode:

Intel Advanced Menu àPCH-IO Configurationà THC configurationà THC1à Enabled

THC MODEà HID Over SPI

Input Report Body Address à[1000]

Input Report Header Addressà[0]

Output Report Address à [4]

Write Opcode à [2]

Read Opcode à[B]

Intel Advanced Menu àPCH-IO Configurationà THC configurationà HID Over SPI Limit Packet Size à[1]

Intel Advanced Menu àPCH-IO Configurationà Security Configurationà Force unlock on all GPIO pads  “Enabled”

BIOS Setting for Dual IO mode:

Flash the latest Released IFWI and do the Below BIOS settings.

- Intel Advanced Menu à PCHIO Configuration àTHC Configuration à THC Port Configuration à THC1 or THC0

- Intel Advanced Menuà PCHIO ConfigurationàTHC Configuration à HID Over SPI Limit Packet Sizeà 1

- Intel Advanced Menuà PCHIO ConfigurationàTHC Configuration à THC Port Configuration àTHC Mode à HID OVER SPI.

- Intel Advanced Menuà PCHIO Configurationà THC Configuration à THC Port Configuration àInput Report Body Addressà [1000]

- Intel Advanced Menuà PCHIO ConfigurationàTHC Configuration à THC Port Configuration à Input Report Header Addressà [0]

- Intel Advanced Menuà PCHIO ConfigurationàTHC Configuration à THC Port Configuration à Output Report Addressà [4]

- Intel Advanced Menuà PCHIO ConfigurationàTHC Configuration à THC Port Configuration à Write Opcodeà [2].

- Intel Advanced Menuà PCHIO Configurationà THC Configuration à THC Port Configuration à Read Opcodeà [BB].

- Intel Advanced Menuà PCHIO ConfigurationàSecurity Configurationà Force Unlock All GPIO Padsà Enabled